Air Force Materiel Command (AFMC) Air Force Nuke Weapons Center (AFNWC) contacted the Scientific Test and Analysis Techniques (STAT) Center of Excellence (COE) at the Air Force Institute of Technology (AFIT) seeking assistance getting more detailed analysis. AFMC AFNWC is working on a reliability analysis regarding emerging failure mechanisms in the Air Launch Cruise Missile (ALCM) power distribution box (J-Box).

Functional tests are performed on several ALCM LRUs (Line Replaceable Unit??) during Analytical Condition Inspection (ACI) at Hill AFB UT. The repair shop at Hill AFB identified excessive corrosion on CR4 diode assembly which led to the ALCM SPO contracting the ALCM OEM, Boeing, to conduct a J-Box aging study. Though we do not have the results1, we were told that Boeing determines that the mounting torque of multiple diodes were below specifications. Corrosion experts at Sandia National Labs later determined that there was condensation caused by capillary action in crimped wires and that the corrosion will continue to increase. The J-Box design includes an uncoated stranded wire splice which does not protect from moisture in surrounding air. Thermal cycles cause expansion and contraction which leads to decreased torque on diode assembly mounting. Operational performance risks are projected to begin when CR4 diode assembly voltage drop exceeds 2.0 direct-current volts (VDC), although Boeing determined that the critical CR4 diode assembly margin between functional test threshold and nominal operational power requirement is 1 additional volt. The Level-III2 test spec for a mounted CR4 diode is 1.0 VDC max. The projected failure mechanism distribution curve for the Level-III functional test indicated an accumulative 10% probability was likely present by 2008, 45% in 2012, and 66% in 2015. The 2015 66% test failure rate represents the accumulated risk that the corrosion failure mechanism will be severe enough to cause test failure of TN 01101 in any given J-Box. In the current environment, the standard wire corrosion is expected to fully mature is approximately 40 years, leading to excessive failure potential. There is a 0.9 volt margin remaining which is expected to reach the nominal operational requirement in 13 years due to an estimated .07 VDC/year drop determined from the Level-I TN 0840 test. The test failure rate distribution curve projected the 13 years to reflect impending risk to operational performance with the risks significantly increasing in 2021.

OUTLINE OF HOW THEY REACHED THEIR RESULTS TO COME

Lingering Questions

What are failures with unknown results representing?

Why are 887 and 1078 removed from the analysis?

Where is the data for the 66% (8 of 12) associated with 2015?

What is the test sample rate? Was all of the population tested?

What is the population?

Can we get the Boeing aging study? Was it done with parts or lab acceleration test?

What does fully mature mean? What determines maturity?

Where are the ACI results?

What do the levels in the test represent?

Level I TN 0840 test data (slide 4) – Where is the data? What does this analysis tell us? What is it showing us?

Is part of the tail number the year it was manufactured? Excel sheet goes back to 1984…

Is “TN” the test number? Is it an error code? Or, is it something else?

We need more data for multiple years